CHX002

DDRPHY

Specification

V0.7

Revision History

| Rev # | Status* | Date | Author | Reason for change/description |
| --- | --- | --- | --- | --- |
| V0.1 | D | 2016/12/1 | Martin Si | Draft version. Copy from T16_CMT003_DDRPHY BYTE Spec V05.docx. Mainly change is: - DCLK structure from PLL to different BYTE |
| V0.2 | U | 2017/3/9 | Martin Si | Draft version. Based on T16_CMT002_DDRPHY BYTE Spec V08.docx. 1. Modify DDR_BYTECLK_BUF_S24 description and clock structure in part 2.3 2. Delete Package description in part 6.4 3. update Bump map 4. add new input pin for latch mode test: LATCHUP_NDTREE_N 5. add new input pin for tuning feedback delay: DLL_FBDLY2~0 6. add frequency change note in chapter 5.6 7. add new input pin for ODT dynamic enable (page 30) : LEADINGOE_X_ODT & LEADINGOE_X_ODT_X4 8. change LEADINGOE_X & LEADINGOE_X_X4 for COMPPD dynamic enable only |
| V0.3 | U | 2017/7/18 | Martin Si | 1. separate DRVMODE_P & DRVMODE_N of DQ/DQS pad, delete pin: DRVMODE_P2~0 DRVMODE_N2~0 add pin: DQ_DRVMODE_P2~0 DQ_DRVMODE_N2~0 DQS_DRVMODE_P2~0 DQS_DRVMODE_N2~0 |
| V0.4 | U | 2017/7/21 | Martin Si | 1. Add pin VREF_MODE2 to separate low/high 4 bit VREFC_CTRL |
| V0.5 | U | 2017/8/10 | Martin Si | 1. delete pin DCLK & DCLK800_IN VCDL_SRCTRL1~0 & VCDL_SFCTRL1~0 2. add pin for input clock DCLKP & DCLKN DCLKP_EN & DCLKN_EN DCLK800P & DCLK800N DCLK800P_EN & DCLK800N_EN BUFRX_BIASEN BUFRX_PTATEN BUFRX_DIGBUFEN BUFRX_IBP100U_IN_0 BUFRX_IBP100U_IN_1 BUFRX_IBP100U_OUT_0 BUFRX_IBP100U_OUT_1 3. add pin for debug DCLKOP_OUT DCLKD_OUT DCLKS_OUT DCLKDSI_OUT DQSN_OUT 4. add pin for Clock Duty Tuning DCLK1600_SRCTRL1~0 DCLK1600_SFCTRL1~0 DCLK800_SRCTRL1~0 DCLK800_SFCTRL1~0 DCLKO_SRCTRL1~0 DCLKO_SFCTRL1~0 DCLKOP_SRCTRL1~0 DCLKOP_SFCTRL1~0 DCLKD_SRCTRL1~0 DCLKD_SFCTRL1~0 DCLKS_SRCTRL1~0 DCLKS_SFCTRL1~0 DCLKDSI_SRCTRL1~0 DCLKDSI_SFCTRL1~0 5. add reserved pin DDRPHY_REV15~DDRPHY_REV0 |
| V0.6 | U | 2017/9/8 | Martin Si | 1. add output pin: DDRPHY_DBGOUT15~0 2. change pin description: DDRPHY_REV0 |
| V0.7 | U | 2017/11/15 | Martin Si | 1. modify pin description DDRMODE1/0 DDRPHY_REV 0/1/2 & 4/5/6 2. add debug table in DDRPHY_DBGOUT description 3. modify external pin of VCDL Loop (DDRPHY_REV0/1/2) 4. update internal constraint table in chapter 5.2.4 & 5.2.2 5. update delay description and add clock balance requirement in Part 2.1 6. update clock domain transfer margin table in 2.2 7. update DDR_BYTECLK_BUF structure in 2.3 |

*Statuses include:  D: Draft,  P: Pending review/frozen,  U: Update needed,  A: Approved

Contents

Revision History	2

1 Introduction	6

1.1 Features	6

1.2 Process	6

1.3 Related Documents	7

2 DDR Top-Level Architecture	8

2.1 DDR Clock Structure Diagram	8

2.2 DDR clock jitter tolerance margin	10

2.3 CHX002 DDR_BYTECLK_BUF_S24	11

3 DDRPHY Architecture	13

3.1 DDRPHY IP List	13

3.2 DDRPHY Clock Scheme	14

3.2.1 TX clock scheme	14

3.2.2 RX Clock Scheme	15

3.3 DDRPHY Block Description	16

3.3.1 VCDL_BYTE_S24	16

3.3.2 DDRCKG_DDRCLK_S24	20

3.3.3 Logic DIOs	21

3.3.4 DDRIO	23

4 DDRPHY Interface List	24

4.1 External DC Supply	24

4.2 External Signal Pins	24

4.2.1 VCDL_BYTE	24

4.2.2 DIO	28

4.2.3 PAD	32

4.3 Internal Connection	39

4.3.1 DDRPHY TXDIO	39

4.3.2 DDRPHY RXDIO	40

4.3.3 DDRPHY DDRCKG	40

4.3.4 DDRPHY DDRIO	40

5 Implement Note	41

5.1 Power states	41

5.1.1 Normal work state	41

5.1.2 DLL WeakLock Mode	41

5.1.3 DCLK turn off	42

5.2 Timing	42

5.2.1 Balance Requirement	42

5.2.2 External Constraint Requirement(less)	47

5.2.3 Internal Constraint Requirement(not final)	47

5.3 Calibration(less)	48

5.4 Glitch	48

6 DDRPHY FloorPlan & BumpMap	50

6.1 Floorplan	50

6.2 Pad assignment	50

6.3 BumpMap	50

7 Test(less)	52

Introduction

Features

- DDRPHY Data & strobe supports for:

- DDR4	X8 1600/1866/2133/2400/2666/3200

- DDR4	X4 1600/1866/2133/2400/2666/3200

- DDR4L	X8 1600/1866/2133/2400/2666/3200

- DDR4L	X4 1600/1866/2133/2400/2666/3200

- Transmitter

- Programmable clock phase for data transfer between different clock domain

- Programmable TX driver output resistance and drive strength

- Programmable Equalizer

- Programmable DQ TX per-bit-de-skew

- Receiver

- Programmable ODT resistance and ODT strength

- Programmable per-bit VREF value

- Programmable RX CTLE

- Programmable DQ RX per-bit-de-skew

- Programmable RX per-bit Strobe phase

- Separate data sampling by rise/fall edge of strobe

-Loading

-Max support 2S4R (2DIMM and 2Rank per DIMM)

Process

Process: TSMC 16nm FFC LL (1P13M_6X2Y2YY2R)

DC Spec

| Corner | Temperature | DVDD | VPP (DDR4) | VPP (DDR4L) |
| --- | --- | --- | --- | --- |
| TT | 85 | 0.8 | 1.2 | 1.05 |
| SS | -40 | 0.72 | 1.14 | 0.945 |
| SS | 125 | 0.72 | 1.14 | 0.945 |
| FF | -40 | 0.88 | 1.26 | 1.155 |
| FF | 125 | 0.88 | 1.26 | 1.155 |

Related Documents

- DDR_BYTECLK_BUF_S24 Specifications

- T16 CHX002 DDR_BYTECLK_BUF layout guide

- T16 CHX002 DDR_PAD Specifications

- T16 CHX002 PLLIN_COMP_ESD Specifications

- T16 CHX002 VCDL_CA Specifications

- T16 DDR Bump Map & Whole Chip Bump map

- CHX002 Pinlist

DDR Top-Level Architecture

DDR Clock Structure Diagram

There are 2 PLL in DDR system and REF clock is their reference clock.

PLLIN will output 1600MHz clock. PLLIN output clock to DRAMC core logic clock tree CTS delay is t1(<1ns) and to PLLIN feedback clock tree delay is also t1. Low frequency jitter in PLLIN bandwidth will be filtered between REF、PLLIN FB and DRAMC leaf clock.

PLLINDDR will output 1600MHz to DDRPHY and DDRIO logic near PHY. PLLINDDR output clock will be balanced through all byte DDRPHY by circuit design and its clock tree delay is t2 (delay~1ns). 1600M clock will be divided by 2 and sample signals from DRAMC in 800M clock domain, the divided clock tree delay is t3 (~100ps). PLLINDDR feedback clock tree delay is t2+t3 (>1ns). Low frequency jitter in PLLINDDR bandwidth will be filtered between REF、PLLINDDR FB and DDRPHY clock.

In the picture below, all the clock phase with red point will be aligned to same clock phase t0.

目前CMT003/CHX002的PLL 和clock structure 修改方案如下。黄色的粗线和绿色的线是需要circuit 做tree的，其他是APR tree。

- PLLIN CKOUT1 1.6G output, CKOUT2 800M output 给logic用，feedback 800M clock,  APR tree t1 feedback 长齐。

- PLLINDDR CKOUT1 1.6G output, CKOUT2 800M output，由circuit 完全一直长到PHY的input，然后PHY 分别output 两个1.6G 和800M clock。这两个clock要求严格balance，skew控制在15+20ps内。这两个clock的绝对latency t2尽量小，在PLL内部和feedback t2 做balance。

- T3是PHY output的APR tree，100ps左右，和PLLDDR 800M feedback外面的t3 balance。

- DIO的逻辑需要经过下面clock domain的转换：

  - PLLIN 800M -> PLLINDDR 800M 1/2T转换(follow Part 2.2.1)

  - PLLINDDR 800M-> PLLINDDR 1600M 转换

  - PLLINDDR 1600M -> PHY VCDL DCLKO 转换(follow Part 2.2.2)

5. Balance requirement

表格中所有的t3需要在TT、SS、FF下尽可能的balance, 总delay在100ps左右。如果不能meet各个corner下都balance，TT的优先级最高

DDR clock jitter tolerance margin

PLLIN and PLLDDR margin

| Item | Description | setup | hold |
| --- | --- | --- | --- |
| phase jitter | PLLIN and PLLDDR 1600M output phase jitter +/-120ps | 120 | 120 |
|  | t1 CTS jitter +/-80ps @2ns (forward+feedback CTS Jitter included) | 80 | 80 |
|  | t2+t3 CTS jitter +/-25ps @500ps | 25 | 25 |
| Period Jitter | PLL output 15ps @1600M，21ps @ 800M(divided clock) | 21 | 0 |
|  | t1 CTS Jper 20ps @2ns | 20 | 0 |
|  | t2 CTS Jper 5ps @500ps | 5 | 0 |
| OCV | 2.1% on launch clock cell 2.1% on capture clock cell 7% on clock net (Worst case t1 2ns, average cell and net 4.5%, DRAMC leaf t1 OCV and PLLIN feedback OCV) | 200 | 0 |
|  | 37ps clock uncertainty | 37 | 0 |
|  | 4.8% on launch clock cell 4.8% on capture clock cell 10% on clock net (best case t1 1ns, average cell and net 7.5%, DRAMC leaf t1 COV and PLLIN feedback OCV) | 0 | 150 |
|  | 38ps clock uncertainty | 0 | 38 |
| SUM2 |  | 508 | 413 |

Clock tree requirement:

      - t0 may be separated to 2 parts. Circuit will do the clock tree to DRAMC core area boundary and to each DDRPHY area boundary in order to reduce t0 total delay. DRAMC internal clock tree will be done by BE tool.

      - t2 path use M13 to transfer DDRPHY clock. Because of big buffer size, enough de-cap should be surrounded and good power connection is must.

DDRPHY DCLK_OUT to DCLKO margin

| Item | Description | setup | hold |
| --- | --- | --- | --- |
| phase jitter | ±20p（PLL高频）±10p（VCDL） | 30 | 30 |
| Period Jitter | ±15p（PLL）±5p（CTS）±10p（VCDL+PI） | 30 | 0 |
| Clock skew | ±5p | 5 | 5 |
| Phase skew DCLKO和理想PH phase之间的skew | ±20p | 20 | 20 |
| Margin | ±25p | 25 | 25 |
| STA uncertainty |  | 110 | 80 |

DDRPHY DCLKO and TXCLKD/S internal margin

| Item | Description | setup | hold |
| --- | --- | --- | --- |
| phase jitter | ±20p（PLL高频）±10p（VCDL） | 30 | 30 |
| Period Jitter | ±15p（PLL）±5p（CTS）±10p（VCDL+PI） | 30 | 0 |
| Clock skew | ±5p | 5 | 5 |
| Phase skew DCLKO/DCLKD 和理想PH phase之间的skew | ±20p | 20 | 20 |
| Margin | ±25p | 25 | 25 |
| STA uncertainty |  | 110 | 80 |

CHX002 DDR_BYTECLK_BUF_S24

In CHX002, 6 DDR_BYTECLK_BUF per channel is used. PLL output clock is buffered (DDR_BYTECLK_BUF) to drive all CA BYTE via M13, then 4 repeater buffer (DDR_BYTECLK_BUF2 in below picture) is introduced to drive all data BYTE. Red/Blue path in picture is 1600M & 800M clock for DDRPHY.

DDRPHY Architecture

DDRPHY IP List

| Group | Basic IP name | Size (before shrink) | Number in PHY | Note |
| --- | --- | --- | --- | --- |
| Analog | VCDL_BYTE_S24 | 606*200 | 1 | TX DQS/DQ clock |
|  | DDRCKG_DDRCLK_S24 | 606*50 | 1 | Clock tree balance |
|  | bdddrdq_S24 | 44.976*500 | 8 | DQ pad |
|  | bdddrdqs_S24 | 89.952*500 | 2 | DQS pad |
|  | padVCC4_MEM_S24 | 44.976*500 | 1 | VCC pad |
|  | Cap Filler | 21.312*500 | 1 |  |
| Digital | DX2DD |  | 8 | TX data |
|  | DX2DS |  | 2 | TX DQS |
|  | TXSDQ |  | 2 | TX data select |
|  | DMOE_GEN |  | 2 | TX output enable |
|  | DQSIEN_GEN |  | 2 | RX input enable |
|  | RXDIO |  | 8 | RX data |

DDRPHY Clock Scheme

TX clock scheme

Note:

      - VCDL_BYTE_S24 input clock frequency is full speed DDR clock

      - All clock generated by VCDL_BYTE_S24 is full speed DDR clock with programmable 64 phase, including DCLKO、DCLKOP、DCLKD、DCLKS、DCLKDSI、DCLKD_X4、DCLKS_X4 and DCLKDSI_X4

      - MD* inputted to DX2DDs is sampled at DCLKO falling edge by external logic IPs. Then DX2DD & DX2DS capture data by DCLKO rising edge.

      - Skew and jitter requirement between different clock please refer to chapter on timing

      - Per-Bit-De-skew is composed by bdddrdq, not in DDRCKG_DDRCLK

| VCDL_BYTE_S24 output clock to DIOs list |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Clock Name DIO Name | DCLKO | DCLKOP | DCLKD & DCLKD_X4 | DCLKS & DCLKS_X4 | DCLKDSI & DCLKDSI_X4 |
| DX2DD | Y |  | Y |  |  |
| DX2DS | Y | Y |  | Y |  |
| TXSDQ | Y | Y |  |  |  |
| DMOE_GEN | Y | Y | Y | Y |  |
| DQSIEN_GEN | Y |  | Y |  | Y |

RX Clock Scheme

Note:

      - DQSOP*/DQSON* for each RXSampler is not of same delay with other RXSampler.

      - Cell TNI_Detector is included in bdddrdqs_S24

      - 2 VCDL_PI_DQS will operate in either X8 or X4 mode, which means connection between RXDIO* to bdddrdq_S24* could not be changed in different X8/X4 mode.

      - 2 DQSIEN_GEN output signal: TNID & TNIS to all DQ/DQS pad input is of same delay.

DDRPHY Block Description

VCDL_BYTE_S24

VCDL_BYTE_S24 is composed of ADLL, 8 MUX_PI, 8 VCDL_PI_DQ, 2 VCDL_PI_DQS and other basic cell. ADLL+MUX_PI(8) would generate 8 clock with 64 phase tunable for DIOs. Both VCDL_PI_DQS will work for X8 or X4 mode. And path from DQS_ORG/DQS_ORG_X4 to each VCDL_PI_DQS input is balanced.

ADLL_S24

ADLL is composed of several basic cells, such as phase detector (PD), charge pump (CP), v2v cell (V2V), analog delay line, and phase interpolator (PI). In general, there is one ADLL in 8’bit DDRPHY.

PD check the phase difference of input clock DCLK and feedback clock output from delay line in ADLL loop, then generate UP/DN signal for CP to increase/decrease the analog delay line control signal: BIASP & BIASN.

- Total 8 full speed clock with 45℃ phase difference with each adjacent clock generated by ADLL will be applied to all TX/RX clock PI

- Analog signal bus: BiasP/BiasN will be used for RX path VCDL_PI_DQS and VCDL_PI_DQ analog delay line too.

Note:

- When DLL_LOCKOK rise to high, it means the delay value of delay line chain is equal to one cycle of DCLK.

- DLL_CKRANGE pin should follow DDR work frequency.

- DLL_REFDLY is used to tuning the VCDL chain delay value.

MUX_PI

MUX_PI is used to generate clock with 64 programmable phase. Each MUX_PI need 9 phase input clock as below picture from phase 0 to phase 360℃. Then 8 phase will be interpolated via either 2 adjacent clock. So each MUX_PI output clock phase can be tuned among 64 step by setting PH_*<5:0>, one step is T/64. For example, below picture illustrates how to generate one phase between 4T/8 and 5T/8, and any clock from 33T/64~39T/64 can be output of MUX_PI according to their setting.

Phase 0T/8  lrhhhflllrhhhflllrhhhfll

Phase 1T/8  llrhhhflllrhhhflllrhhhfl

Phase 2T/8  lllrhhhflllrhhhflllrhhhf

Phase 4T/8  llllrhhhflllrhhhflllrhhh

Phase 5T/8  lllllrhhhflllrhhhflllrhh

Phase 6T/8  llllllrhhhflllrhhhflllrh

Phase 7T/8  lllllllrhhhflllrhhhflllr

Phase 8T/8  llllllllrhhhflllrhhhflll

Phase 4T/8    lrhhhhhhhhhhhhhhhhhhhhh

Phase 33T/64  llrhhhhhhhhhhhhhhhhhhhh

Phase 34T/64  lllrhhhhhhhhhhhhhhhhhhh

Phase 35T/64  llllrhhhhhhhhhhhhhhhhhh

Phase 36T/64  lllllrhhhhhhhhhhhhhhhhh

Phase 37T/64  llllllrhhhhhhhhhhhhhhhh

Phase 38T/64  lllllllrhhhhhhhhhhhhhhh

Phase 39T/64  llllllllrhhhhhhhhhhhhhh

Phase 5T/8    lllllllllrhhhhhhhhhhhhh

VCDL_PI_DQS_S24

VCDL_PI_DQS is composed of VCDL (voltage controlled delay line) chain and several MUX_PI for DQSOP* and DQSON*, which is input clock of RXSampler in RXDIO.

VCDL can work only after DLL_LOCKOK rise to high. The delay value of each VCDL cell is T/8, controlled by BiasP & BiasN from ADLL. There are total 8 VCDL cell and 9 clock from phase 0 to phase 360℃ by step 45℃ would be generated as input clock of all PI in VCDL_PI_DQS.

Each PI would generate corresponding clock with phase programmable range from 0 to 1T by step T/64. And RX work range of each MUX_PI may diff with other.

There are 8 MUX_PI in one VCDL_PI_DQS_S24, and total 16 MUX_PI for RX path in one DDRPHY.

VCDL_PI_DQ_S24

VCDL_PI_DQS is composed of VCDL chain and MUX, which is used to coarse tuning de-skew between different DQ in one BYTE. DQ can delay by 5 setting, which is 0,T/8,T/4,3T/8, and T/2.

DDRCKG_DDRCLK_S24

Logic DIOs

TXSDQ

DX2DD

DX2DS

DMOE_GEN

DQSIEN_GEN

RXDIO

DDRIO

DDRIO list as below, and please refer to T16 CHX002 DDR PAD spec* for detail.

Bdddrdq_S24 ( total 8)

bdddrdqs_S24 (total 2)

padVCC4_MEM_S24 (total 1)

DDRPHY Interface List

External DC Supply

| Signal Name | Domain | I/O | Description |
| --- | --- | --- | --- |
| DVDD | Core | Power | Core power supply |
| DVSS | Ground | Ground | Ground for core & IO & RVPP domain |
| VPP | VPP | Power | IO pad power supply, typical 1.2V for DDR4 & 1.05V for DDR4L |

External Signal Pins

BUFRX for clock

| Signal Name | Power Domain | I/O | Description |
| --- | --- | --- | --- |
| DCLKP DCLKN | CORE | I | Input full speed differential clock from PLL, 800MHz~1600MHz |
| DCLK800P DCLK800N | CORE | I | Input half speed differential clock from PLL 400MHz~800MHz |
| DCLKP_EN DCLKN_EN | CORE | I | DCLKP & DCLKN enable 1: enable DCLKP & DCLKN(default) 0: disable DCLKP & DCLKN |
| DCLK800P_EN DCLK800N_EN | CORE | I | DCLK800P & DCLK800N enable 1: enable DCLK800P & DCLK800N(default) 0: disable DCLK800P & DCLK800N |
| DCLK1600_SRCTRL1 DCLK1600_SRCTRL0 DCLK1600_SFCTRL1 DCLK1600_SFCTRL0 | Core | I | Duty tuning for DCLK1600 input clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| DCLK800_SRCTRL1 DCLK800_SRCTRL0 DCLK800_SFCTRL1 DCLK800_SFCTRL0 | Core | I | Duty tuning for DCLK800 input clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| BUFRX_BIASEN | CORE | I | Enable DCLKP/N & DCLK800P/N bias current 1: enable(default) 0:disable |
| BUFRX_PTATEN | CORE | I | select current of PTAT or BUFRX_IBP100U_IN 1: PTAT current 0: BUFRX_IBP100U_IN_0/1 current |
| BUFRX_DIGBUFEN | CORE | I | DCLKP/N & DCLK800P/N mode 0: analog mode(default) 1: digital mode |
| BUFRX_IBP100U_IN_0 BUFRX_IBP100U_IN_1 | CORE | I | 100uA bias current input |
| BUFRX_IBP100U_OUT_0 BUFRX_IBP100U_OUT_1 | CORE | O | 100uA bias current output |
| DDRPHY_REV15 DDRPHY_REV14 DDRPHY_REV13 DDRPHY_REV12 DDRPHY_REV11 DDRPHY_REV10 DDRPHY_REV9 DDRPHY_REV8 DDRPHY_REV7 DDRPHY_REV3 | CORE | I | DDRPHY_REV3 & 7~15 Reserved |
| DDRPHY_REV6 DDRPHY_REV5 DDRPHY_REV4 |  |  | Debug signal control. Refer debug table in DDRPHY_DBGOUT descrition. |
| DDRPHY_REV2 DDRPHY_REV1 | CORE | I | used for de-glitch enable of TX clock phase change ( DCLKO/DCLKOP/DCLKD/DCLKS/DCLKDSI) REV2:1=11: enable de-glitch, 2T loss REV2:1=10: enable de-glitch, 1T loss REV2:1=00: disable de-glitch REV2:1=01: Reserved (Same description in VCDL_BYTE) |
| DDRPHY_REV0 | CORE | I | Connect to VCDL_BYTE DLL_CP_EN, used to enable bias current advanced by DLL_EN. 1: always enable bias 0: enable bias only when DLL_EN=0 (Same description in VCDL_BYTE) |
| DDRPHY_DEBUGOUT15 DDRPHY_DEBUGOUT14 DDRPHY_DEBUGOUT13 DDRPHY_DEBUGOUT12 DDRPHY_DEBUGOUT11 DDRPHY_DEBUGOUT10 DDRPHY_DEBUGOUT9 DDRPHY_DEBUGOUT8 DDRPHY_DEBUGOUT7 DDRPHY_DEBUGOUT6 DDRPHY_DEBUGOUT5 DDRPHY_DEBUGOUT4 DDRPHY_DEBUGOUT3 DDRPHY_DEBUGOUT2 DDRPHY_DEBUGOUT1 DDRPHY_DEBUGOUT0 | CORE | O | Debug signal output Debug table as below. |

|  | DDRPHY_REV4=1 | DDRPHY_REV5=1 | DDRPHY_REV6=1 |
| --- | --- | --- | --- |
| DDRPHY_DEBUGOUT15 | TNI_GATE | TNI_GATE | TNI_GATE |
| DDRPHY_DEBUGOUT14 |  |  | Bit6 TE15 |
| DDRPHY_DEBUGOUT13 |  |  | Bit6 TE0 |
| DDRPHY_DEBUGOUT12 |  |  | RSTEN |
| DDRPHY_DEBUGOUT11 |  | DSODT | MDI6 |
| DDRPHY_DEBUGOUT10 |  | CMPENDQS | DQSP6 |
| DDRPHY_DEBUGOUT9 |  | CMPENDQ | DQSN6 |
| DDRPHY_DEBUGOUT8 |  | DQIEN | DQSIEN_HEAD2 |
| DDRPHY_DEBUGOUT7 |  | DSIEN | DQSIEN_HEAD0 |
| DDRPHY_DEBUGOUT6 |  | DQODT | DDWPG2 |
| DDRPHY_DEBUGOUT5 | DL_DQS of DX2DS |  |  |
| DDRPHY_DEBUGOUT4 | DH_DQS of DX2DS |  |  |
| DDRPHY_DEBUGOUT3 | TNOS of DMOE_GEN |  |  |
| DDRPHY_DEBUGOUT2 | TNOD of DMOE_GEN |  |  |
| DDRPHY_DEBUGOUT1 | DLO of DX2DD0 |  |  |
| DDRPHY_DEBUGOUT0 | DHO of DX2DD0 |  |  |

VCDL_BYTE

ADLL_S24+MUX_PI for TX

| Signal Name | Power Domain | I/O | Description |
| --- | --- | --- | --- |
| DLL_EN | CORE | I | ADLL enable DLL_EN=1, ADLL enable DLL_EN=0, ADLL disable |
| DDRPHY_REV0 | CORE | I | used to enable bias current advanced by DLL_EN. 1: always enable bias, large power 0: enable bias only when DLL_EN=0 |
| DDRPHY_REV2 DDRPHY_REV1 | CORE | I | used for de-glitch enable of TX clock phase change ( DCLKO/DCLKOP/DCLKD/DCLKS/DCLKDSI) REV2:1=11: enable de-glitch, 2T loss REV2:1=10: enable de-glitch, 1T loss REV2:1=00: disable de-glitch REV2:1=01: Reserved |
| VCDL_BYPASS | CORE | I | VCDL_BYPASS=1: test mode to bypass delay line VCDL_BYPASS=0: normal work mode |
| DLL_CKRANGE2 DLL_CKRANGE1 DLL_CKRANGE0 | CORE | I | Select corresponding setting for different DCLK frequency 3’b000~3’b011: reserved 3’b 100:400M~565M 3’b 101:565M~800M 3’b 110:800M~1.13G 3’b 111:1.13G~1.6G |
| DLL_REFDLY2 DLL_REFDLY1 DLL_REFDLY0 | CORE | I | 3 bit delay setting to correct reference clock delay line in ADLL loop Default: 3’b011 |
| DLL_FBDLY2 DLL_FBDLY1 DLL_FBDLY0 | CORE | I | 3 bit delay setting to correct feedback delay line in ADLL loop Default: 3’b011 |
| DLL_V2VSEL2 DLL_V2VSEL1 DLL_V2VSEL0 | CORE | I | Tuning voltage range of BIASP/BIASN for analog delay line Default:3’b100 |
| DLL_BWSEL1 DLL_BWSEL0 | CORE | I | Tuning charge current value for charge pump in ADLL to tuning bandwidth Default:2’b01 |
| DLL_WEAKLOCK | CORE | I | Weak lock control for lower power. When DLL_WEAKLOCK asserted, ADLL loop is disabled and BIASP/BIASN will change slow by leakage. DLL_WEAKLOCK should be de-assert after given resume time. DLL_WEAKLOCK=1, weak lock enable DLL_WEAKLOCK=0, weak lock disable (Note: resume time may be 0.1~1us, not final) |
| DLL_WEAKLOCK_GATE | Core | I | Gating DCLK when DLL_WEAKLOCK enable DLL_WEAKLOCK_GATE=1，gate DCLK DLL_WEAKLOCK_GATE=1，not gate DCLK |
| PH_DCLKO5 PH_DCLKO4 PH_DCLKO3 PH_DCLKO2 PH_DCLKO1 PH_DCLKO0 | CORE | I | Phase setting for output clock: DCLKO 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKOP5 PH_DCLKOP4 PH_DCLKOP3 PH_DCLKOP2 PH_DCLKOP1 PH_DCLKOP0 | CORE | I | Phase setting for output clock: DCLKOP 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKD5 PH_DCLKD4 PH_DCLKD3 PH_DCLKD2 PH_DCLKD1 PH_DCLKD0 | CORE | I | Phase setting for output clock: DCLKD 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKD5_X4 PH_DCLKD4_X4 PH_DCLKD3_X4 PH_DCLKD2_X4 PH_DCLKD1_X4 PH_DCLKD0_X4 | CORE | I | Phase setting for output clock: DCLKD_X4 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKS5 PH_DCLKS4 PH_DCLKS3 PH_DCLKS2 PH_DCLKS1 PH_DCLKS0 | CORE | I | Phase setting for output clock: DCLKS 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKS5_X4 PH_DCLKS4_X4 PH_DCLKS3_X4 PH_DCLKS2_X4 PH_DCLKS1_X4 PH_DCLKS0_X4 | CORE | I | Phase setting for output clock: DCLKS_X4 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKDSI5 PH_DCLKDSI4 PH_DCLKDSI3 PH_DCLKDSI2 PH_DCLKDSI1 PH_DCLKDSI0 | CORE | I | Phase setting for output clock: DCLKDSI 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PH_DCLKDSI5_X4 PH_DCLKDSI4_X4 PH_DCLKDSI3_X4 PH_DCLKDSI2_X4 PH_DCLKDSI1_X4 PH_DCLKDSI0_X4 | CORE | I | Phase setting for output clock: DCLKDSI_X4 6’b000000: Phase 0 …. 6’b111111: Phase 63 |
| PIEN_DCLKO | CORE | I | Output clock enable: DCLKO |
| PIEN_DCLKOP | CORE | I | Output clock enable: DCLKOP |
| PIEN_DCLKD | CORE | I | Output clock enable: DCLKD |
| PIEN_DCLKD_X4 | CORE | I | Output clock enable: DCLKD_X4 |
| PIEN_DCLKS | CORE | I | Output clock enable: DCLKS |
| PIEN_DCLKS_X4 | CORE | I | Output clock enable: DCLKS_X4 |
| PIEN_DCLKDSI | CORE | I | Output clock enable: DCLKDSI |
| PIEN_DCLKDSI_X4 | CORE | I | Output clock enable: DCLKDSI_X4 |
| DLL_LOCKOK | CORE | O | ADLL loop lock OK flag DLL_LOCKOK =1 means ADLL loop lock and output clock is valid DLL_LOCKOK =0 means ADLL loop is unlocked and DDRPHY can’t work |
| DLL_FALSELOCK_DEBUG1 DLL_FALSELOCK_DEBUG0 | CORE | O | Debug signal for ADLL false lock |
| DLL_CKPH_DEBUG8 DLL_CKPH_DEBUG7 DLL_CKPH_DEBUG6 DLL_CKPH_DEBUG5 DLL_CKPH_DEBUG4 DLL_CKPH_DEBUG3 DLL_CKPH_DEBUG2 DLL_CKPH_DEBUG1 DLL_CKPH_DEBUG0 | CORE | O | Debug clock for ADLL loop clock phase output (high speed) |
| DLL_CLK1600 | CORE | O | Debug clock for buffered full speed input PLL clock |
| DLL_CLK800 | CORE | O | Debug clock for buffered half speed input PLL clock |

DDRCKG+RX VCDL_PI_ DQS + VCDL_PI_DQ

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| RDDRX4 | I | Core | DDR X4 mode setting 1’b1 X4 mode 1’b0 X8 mode |
| DCLKO_SRCTRL1 DCLKO_SRCTRL0 DCLKO_SFCTRL1 DCLKO_SFCTRL0 | I | Core | Duty tuning for DCLKO output clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| DCLKOP_SRCTRL1 DCLKOP_SRCTRL0 DCLKOP_SFCTRL1 DCLKOP_SFCTRL0 | I | Core | Duty tuning for DCLKOP output clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| DCLKD_SRCTRL1 DCLKD_SRCTRL0 DCLKD_SFCTRL1 DCLKD_SFCTRL0 | I | Core | Duty tuning for DCLKD output clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| DCLKS_SRCTRL1 DCLKS_SRCTRL0 DCLKS_SFCTRL1 DCLKS_SFCTRL0 | I | Core | Duty tuning for DCLKS output clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| DCLKDSI_SRCTRL1 DCLKDSI_SRCTRL0 DCLKDSI_SFCTRL1 DCLKDSI_SFCTRL0 | I | Core | Duty tuning for DCLKDSI output clock 20ps per step @ss -40C as below SRCTRL; SFCTRL; Rise delay; Fall delay. 11; 00; +60; +0. 10; 00; +40; +0. 01; 00; +20; +0. 00; 00; +0; +0. 00; 01; +0; +20. 00; 10; +0; +40. 00; 11; +0; +60. |
| PH_DQ[7:0]_[2:0] | I | Core | DQ[7:0] per-bit-deskew, coarse tuning DQ Delay from MDI[7:0] to MDO[7:0] increases by step=T/8 from code 000 to code 100 Code 101~111 reserved |
| PH_DQS[7:0]P[5:0] | I | Core | DQSOP0[7:0] phase selection, fine tuning Phase step is T/64 from code 0 to code 6’b111111 |
| PH_DQS[7:0]N[5:0] | I | Core | DQSON0[7:0] phase selection, fine tuning Phase step is T/64 from code 0 to code 6’b111111 |
| PIEN_DQS PIEN_DQS_X4 | I | Core | RX DQS PI enable for X8/X4 mode 1: enable (default) 0: disable |
| MUXEN_DQ7 MUXEN_DQ6 MUXEN_DQ5 MUXEN_DQ4 MUXEN_DQ3 MUXEN_DQ2 MUXEN_DQ1 MUXEN_DQ0 | I | Core | RX DQ MUX_DQ enable for DQ[7:0] 1: enable (default) 0: disable |
| DQSP_OUT | O | CORE | RX DQSP1 output debug signal |
| DQSN_OUT | O | CORE | RX DQSN1 output debug signal |

DIO

DCLKO_OUT

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| DCLKO_OUT | CORE | O | 64 phase programmable clock to controller near PHY, it is sync with internal clock of TXDIOs |
| DCLK_OUT | CORE | O | Buffed DCLK to logic |
| DCLK800_OUT | CORE | O | Buffed DCLK800_IN to logic |

TXSDQ (Number=2)

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| WRDY_IN WRDY_IN_X4 | I | Core |  |

DX2DD (Number=8)

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| MDL0_[7:0] | I | Core |  |
| MDL1_[7:0] | I | Core |  |
| MDH0_[7:0] | I | Core |  |
| MDH1_[7:0] | I | Core |  |
| TESTEN_7 TESTEN_6 TESTEN_5 TESTEN_4 TESTEN_3 TESTEN_2 TESTEN_1 TESTEN_0 | I | Core |  |
| TESTIN_7 TESTIN_6 TESTIN_5 TESTIN_4 TESTIN_3 TESTIN_2 TESTIN_1 TESTIN_0 | I | Core |  |

DX2DS (Number=2)

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| SDHXX_PRE SDHXX_PRE_X4 | I | Core |  |
| SDLXX_PRE SDLXX_PRE_X4 | I | Core |  |
| TESTIN_DQS TESTIN_DQS_X4 | I | Core |  |
| TESTEN_DQS TESTEN_DQS_X4 | I | Core |  |

DMOE_GEN (Number=2)

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| MDOEDDX MDOEDDX_X4 | I | Core |  |
| DQSOEDDX_PRE DQSOEDDX_PRE_X4 | I | Core |  |
| DQWPH1_X8 DQWPH0_X8 DQWPH1_X4 DQWPH0_X4 | I | Core |  |
| DSWPH1_X8 DSWPH0_X8 DSWPH1_X4 DSWPH0_X4 | I | Core |  |
| RST_ | I | Core | RST_ again，who control？ |
| DRAMDATAOE_EN | I | Core |  |
| WLVL_MODE | I | Core |  |
| RDSOLNGPRE1 RDSOLNGPRE0 | I | Core |  |
| DQTESTEN | I | Core | X4 need or not？ |
| DSTESTEN | I | Core | X4 need or not？ |
| TEST_TNOD | I | Core |  |
| TEST_TNOS | I | Core |  |

DQSIEN_GEN(Number=2)

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| DQSIEN_HEAD0X DQSIEN_HEAD0X_X4 | I | Core |  |
| DQSIEN_HEAD2X DQSIEN_HEAD2X_X4 | I | Core |  |
| LEADINGOE_X LEADINGOE_X_X4 | I | Core |  |
| LEADINGOE_X_ODT LEADINGOE_X_ODT_X4 | I | Core |  |
| DDWPG2_X DDWPG2_X_X4 | I | Core |  |
| DSIEN1_X8 DSIEN0_X8 DSIEN1_X4 DSIEN0_X4 | I | Core |  |
| DMRLEN_RST_ | I | Core |  |
| RST_ | I | Core | 与其他的RST_是否相同 |
| RDSIEN | I | Core |  |
| RDQIEN | I | Core |  |
| FLOATDSI | I | Core | X4 need or not? |
| FLOATDQI | I | Core | X4 need or not? |
| RCMPEN_ALWSON | I | Core |  |
| RCMPENDQ_ON | I | Core |  |
| RCMPENDQS_ON | I | Core |  |
| RDSODTON | I | Core |  |
| RDQODTON | I | Core |  |
| RODTEN | I | Core |  |
| ATPGEN | I | Core |  |
| LATCHUP_NDTREE_N | I | CORE | DQ/DQS COMPPD control in Latch up / NDTREE mode 0: DQ/DQS COMPPD=0 1: normal read/write mode |

RXDIO（Number=8）

| Signal Name | Direction | Power Domain | Description |
| --- | --- | --- | --- |
| DMRLEN_RST_ | I | Core |  |
| RXSAMPLER_EN | I | Core |  |
| CNT_DQSOPN1 CNT_DQSOPN0 | I | Core |  |
| RST_ | I | Core |  |
| LD0F_b[7:0] LD1F_b[7:0] LD2F_b[7:0] LD3F_b[7:0] LD4F_b[7:0] LD5F_b[7:0] LD6F_b[7:0] LD7F_b[7:0] | O | Core |  |
| LD0R_b[7:0] LD1R_b[7:0] LD2R_b[7:0] LD3R_b[7:0] LD4R_b[7:0] LD5R_b[7:0] LD6R_b[7:0] LD7R_b[7:0] | O | Core |  |

PAD

bdddrdq (Number=8)

| Signal Name | I/O | Power Domain | Description |  |
| --- | --- | --- | --- | --- |
| DQ_NS3 DQ_NS2 DQ_NS1 DQ_NS0 | I | core | Four bits driving setting for nmos pulling down 4’b0000 max output resistance … 4’b1111 min output resistance |  |
| DQ_PS3 DQ_PS2 DQ_PS1 DQ_PS0 | I | core | Four bits driving setting for pmos pulling up 4’b0000 max output resistance … 4’b1111 min output resistance |  |
| FTB_DET | I | VPP | Feed through SUS mode power down control. TX / RX / ODT all disable, IO high-Z when FTB_DET=1 |  |
| DLY_DQ[7:0]3 DLY_DQ[7:0]2 DLY_DQ[7:0]1 DLY_DQ[7:0]0 | I | Core | Gray code TX Per-Bit-Deskew setting, total 16 step |  |
| IO7 IO6 IO5 IO4 IO3 IO2 IO1 IO0 | IO | VPP | IO port to board |  |
| DQ_DRVMODE_N2 DQ_DRVMODE_N1 DQ_DRVMODE_N0 | I | core | 000 | Output pull down 240ohm, Rnom/1 |
|  |  |  | 001 | Output pull down 120ohm, Rnom/2 |
|  |  |  | 010 | Output pull down 80ohm, Rnom/3 |
|  |  |  | 011 | Output pull down 60ohm, Rnom/4 |
|  |  |  | 100 | Output pull down 48ohm, Rnom/5 |
|  |  |  | 101 | Output pull down 40ohm, Rnom/6 |
|  |  |  | 110 | Output pull down 34ohm, Rnom/7 |
|  |  |  | 111 | Reserve |
| DQ_DRVMODE_P2 DQ_DRVMODE_P1 DQ_DRVMODE_P0 | I | core | 000 | Output pull up 240ohm, Rnom/1 |
|  |  |  | 001 | Output pull up 120ohm, Rnom/2 |
|  |  |  | 010 | Output pull up 80ohm, Rnom/3 |
|  |  |  | 011 | Output pull up 60ohm, Rnom/4 |
|  |  |  | 100 | Output pull up 48ohm, Rnom/5 |
|  |  |  | 101 | Output pull up 40ohm, Rnom/6 |
|  |  |  | 110 | Output pull up 34ohm, Rnom/7 |
|  |  |  | 111 | Reserve |
| ODTMODE2 ODTMODE1 ODTMODE0 | I | core | 000 | RODTpu/pd=240ohm, Rnom/1 |
|  |  |  | 001 | RODTpu/pd=120ohm, Rnom/2 |
|  |  |  | 010 | RODTpu/pd=80ohm, Rnom/3 |
|  |  |  | 011 | RODTpu/pd=60ohm, Rnom/4 |
|  |  |  | 100 | RODTpu/pd=48ohm, Rnom/5 |
|  |  |  | 101 | RODTpu/pd=40ohm, Rnom/6 |
|  |  |  | 110 | RODTpu/pd=34ohm, Rnom/7 |
|  |  |  | 111 | Reserve |
| DDRMODE1 DDRMODE0 | I | core | 00 | DDR3 1.35V, Rodt pu & pd both |
|  |  |  | 01 | LPDDR4 1.05V, RODT pd only |
|  |  |  | 10 | DDR4L 1.05V, RODT pu only |
|  |  |  | 11 | DDR4 1.2V, RODT pu only |
| DE1_EN DE2_EN | I | Core | 00 | De_emphasis disable |
|  |  |  | 01 | Two former bit de_emphasis enable |
|  |  |  | 10 | Former bit de_emphasis enable |
|  |  |  | 11 | Two former bit de_emphasis and Former bit de_emphasis all enable |
| EQ_SET_N3 EQ_SET_N2 EQ_SET_N1 EQ_SET_N0 | I | Core | 0000~1000 | DQ TX Pull down emphasis strength 0000 minimum … 1000 maximum |
|  |  |  | 1001~1111 | Reserved |
| EQ_SET_P3 EQ_SET_P2 EQ_SET_P1 EQ_SET_P0 | I | Core | 0000~1000 | DQ TX Pull up emphasis strength 0000 minimum … 1000 maximum |
|  |  |  | 1001~1111 | Reserved |
| VREF_SEL | IN | core | NA | VREF selection. 1: VREF_INT 0: VREF_EXT Default:1 |
| NDTREE | In | core | NA | NAND tree enable. (High active) |
| PI[7] PI[6] PI[5] PI[4] PI[3] PI[2] PI[1] PI[0] | In | core | NA | NAND tree input. |
| PO[7] PO[6] PO[5] PO[4] PO[3] PO[2] PO[1] PO[0] | Out | core | NA | NAND tree output. |
| BIASEN | In | Core | NA | DQ/DQS RX bias current enable 1: enable (default) 0: disable |
| CTLE_EN | In | Core | NA | RX EQ enable 1:enable 0:disable |
| CTLE_TUNE5 CTLE_TUNE4 CTLE_TUNE3 CTLE_TUNE2 CTLE_TUNE1 CTLE_TUNE0 | In | Core |  | RX EQ tuning |
| RXMODE1 RXMODE0 | in | Core | 11 | For max Ron and min Rtt in DDR4 |
|  |  |  | 10 | For min Ron and max Rtt in DDR4 |
|  |  |  | 00 | For min Ron and max Rtt in DDR4L |
|  |  |  | 01 | For max Ron and min Rtt in DDR4L |
| ZIT7 ZIT6 ZIT5 ZIT4 ZIT3 ZIT2 ZIT1 ZIT0 | Out | core | NA | Output port to core, for test, driving same as ZI. ZIT=IO when COMPPD=1&TNI=1 ZIT=0 when COMPPD=0 or TNI=0 |
| DDR_PAD_INLP | IN | Core | 1 | SEL=1： ZI=DH SEL=0： ZI=DL |
|  |  |  | 0 | COMPPD=1 & TNI=1： ZI=IO Else： ZI=0 |
| VREFF[7:0]_CTRL[3:0] | In | Core | NA | Fine Tuning for per-bit VREF In X8 mode, VREFF[7:0]_CTRL[3:0] share same range according to VREFC_CTRL[5:0], there is 16 step for per-DQ-VREF In X4 mode, REFF[3:0]_CTRL[3:0] share same range according to VREFC_CTRL[5:0], and REFF[7:4]_CTRL[3:0] share same range according to VREFC_CTRL[5:0]_X4. Please refer to DDRIO padVCC4_MEM spec for detail |
| VREF_EXT | IO | VPP | NA | External VREF from PCB, which value is 50% VPP |

bdddrdqs (Number=2)

| Pin name | In/Out | domain | Value | Description |
| --- | --- | --- | --- | --- |
| DQS_NS3 DQS_NS2 DQS_NS1 DQS_NS0 | In | Core | NA | Four bits driving setting for nmos pulling down |
| DQS_PS3 DQS_PS2 DQS_PS1 DQS_PS0 | In | Core | NA | Four bits driving setting for pmos pulling up |
| EQS_SET_N3 EQS_SET_N2 EQS_SET_N1 EQS_SET_N0 | I | Core | 0000~1000 | DQS TX Pull down emphasis strength 0000 minimum … 1000 maximum |
|  |  |  | 1001~1111 | Reserved |
| EQS_SET_P3 EQS_SET_P2 EQS_SET_P1 EQS_SET_P0 | I | Core | 0000~1000 | DQS TX Pull up emphasis strength 0000 minimum … 1000 maximum |
|  |  |  | 1001~1111 | Reserved |
| DQS_DRVMODE_N2 DQS_DRVMODE_N1 DQS_DRVMODE_N0 | I | core | 000 | Output pull down 240ohm, Rnom/1 |
|  |  |  | 001 | Output pull down 120ohm, Rnom/2 |
|  |  |  | 010 | Output pull down 80ohm, Rnom/3 |
|  |  |  | 011 | Output pull down 60ohm, Rnom/4 |
|  |  |  | 100 | Output pull down 48ohm, Rnom/5 |
|  |  |  | 101 | Output pull down 40ohm, Rnom/6 |
|  |  |  | 110 | Output pull down 34ohm, Rnom/7 |
|  |  |  | 111 | Reserve |
| DQS_DRVMODE_P2 DQS_DRVMODE_P1 DQS_DRVMODE_P0 | I | core | 000 | Output pull up 240ohm, Rnom/1 |
|  |  |  | 001 | Output pull up 120ohm, Rnom/2 |
|  |  |  | 010 | Output pull up 80ohm, Rnom/3 |
|  |  |  | 011 | Output pull up 60ohm, Rnom/4 |
|  |  |  | 100 | Output pull up 48ohm, Rnom/5 |
|  |  |  | 101 | Output pull up 40ohm, Rnom/6 |
|  |  |  | 110 | Output pull up 34ohm, Rnom/7 |
|  |  |  | 111 | Reserve |
| PI_DQS_P PI_DQS_N PI_DQS_P_X4 PI_DQS_N_X4 | In | Core | NA | DQS、DQS_X4 NAND Tree input |
| PO_DQS_P PO_DQS_N PO_DQS_P_X4 PO_DQS_N_X4 | Out | Core | NA | DQS、DQS_X4 NAND Tree output |
| IOP ION | IO | VPP | NA | X4 DQSP/N IO port to board. |
| IOP_X4 ION_X4 | IO | VPP | NA | X8 & X4[3:0] DQSP/N IO port to board. |
| ZITP ZITP_X4 | Out | core | NA | Output port to core, for test, driving same as ZI. ZITP=IOP when COMPPD=1&TNI=1 ZITP=0 when COMPPD=0 or TNI=0 |
| TNI_SEL | In | Core | NA | DQS pre-amble calibration range selection: 1: TNI calibration DQS High 0: TNI calibration DQS Low |

padVCC4_MEM (Number=1)

| Pin name | In/Out | domain | Description |
| --- | --- | --- | --- |
| PD_BIAS | In | core | Powerdown control of Ibias. 1: enable bias 0: disable bias Settle time of IBIAS is 100ns. |
| VREFC_CTRL5 VREFC_CTRL4 VREFC_CTRL3 VREFC_CTRL2 VREFC_CTRL1 VREFC_CTRL0 | In | core | VREF coarse control for per-bit-VREF for DQ[7:0] at VREF_MODE2=0 or DQ[3:0] at VREF_MODE=1 |
| VREFC_CTRL5_X4 VREFC_CTRL4_X4 VREFC_CTRL3_X4 VREFC_CTRL2_X4 VREFC_CTRL1_X4 VREFC_CTRL0_X4 | In | core | VREF coarse control for per-bit-VREF for DQ[7:4] at VREF_MODE2=1 |
| VREF_MODE | In | core | VREF mode selection 1 for coarse mode , step larger 0 for fine mode, step 0.0045V |
| VREF_MODE2 | In | Core | Low/High 4 bit VREF control mode 1: Low/High 4 bit use separate VREFC_CTRL5~0 0: Low/High 4 bit use same VREFC_CTRL5~0 |
| VREF_OVERLAP | In | core | Overlap mode for VREF calibration |
| VREF_OVERLAP_X4 | In | core | Overlap mode for VREF calibration |
| PD_VREF | In | Core | Powerdown control of VREF 1: enable VREF 0: disable VREF Settle time of VREF is 1us. |
| VREF_TST2 VREF_TST1 VREF_TST0 | In | Core | VREF for test 000: VREFC_H 001: VREFC_L 010: VPP 011: DVDD 100: DVSS |
| IREF_TST1 IREF_TST0 | in | Core | IREF for test 01: IBN 10: IBP |
| VREF_TST_EN | in | Core | Test mode enable 1:enable |

Internal Connection

DDRPHY TXDIO

TXSDQ to DX2DD & DX2DS

SEL_H/SEL_L & SEL_H_X4/SEL_L_X4 from TXSDQ/TXSDQ_X4 connect to DX2DD[7:0] and DX2DS & DX2DS_X4 as below.

Timing requirement：

            - SEL_HSEL_LSEL_H_X4SEL_L_X4 balance with each other

            - Each SEL should follow 5T/8 setup & -T/8 hold timing

DMOE_GEN to DX2DD & DX2DS

DDRPHY RXDIO

Tree for DQSIEN_HEAD0, DQSIEN_HEAD2, DDWPG2

DDRPHY DDRCKG

Follow DDRCKG_DDRCLK_S24

TNOD/TNOS/TNOD_X4/TNOS_X4

TNID/TNIS/TNID_X4/TNIS_X4

DCLKO/DCLKOP

DCLKD/DCLKS/DCLKD_X4/DCLKS_X4

DDRPHY DDRIO

COMPPD/ODT/NDTREE and others

Implement Note

Power states

Normal work state

After DLL_EN asserts, ADLL need more than 8000 cycle to settle stable delay (total 1T in loop) with core power and PLL clock ready. If DLL_LOCKOK doesn’t turns to logic High, it means different clock phase generated by ADLL and RX delay line is not accurate, and all logic IP can’t work normally.

In below picture, t1>0, t2>0 and t3>8000T

DLL WeakLock Mode

Assert DLL_WEAKLOCK will enable power saving mode, and DDRPHY could resume immediately when de-assert DLL_WEAKLOCK. In this mode, DCLK will be gated by DLL_WEAKLOCK, and delay of analog delay line will hold in certain time. Then DLL_WEAKLOCK should be de-asserted and ADLL loop re-work to correct the delay of delay line. In Weaklock mode, DLL_LOCKOK is always on.

As below diagram, when DLL_WEAKLOCK turns to 1, delay of delay line will change slowly. Delay will change to T+td in t4 time. Then controller turn off DLL_WEAKLOCK mode, and delay will change back to T in t5 time.

In CMT003 DDRPHY, td is limited to 5% of T, such as for 1600MHz clock, td=31.25ps. In t5 time or later, DDRPHY can work fine.

td<5%T

t4<1us

t5>100ps

DCLK turn off

WEAKLOCK mode could not be applied to case as below:

- DVDD power down, suspend mode as example

- DCLK is turn off for longer time than t4.

Once they happen, DLL_EN should be de-asserted and it takes t3 time to resume DDRPHY after DLL_EN assert again.

Timing

Balance Requirement

Path 1 SEL_H/L & SEL_H/L_X4 from TXSDQ to DX2DD & DX2DS

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| TXSDQ | SH | DX2DD0 DX2DD1 DX2DD2 DX2DD3 DX2DD4 DX2DD5 DX2DD6 DX2DD7 | SEL_H | T1 | T/2 setup & –T/4 hold time requirement |
|  | SL | DX2DD0 DX2DD1 DX2DD2 DX2DD3 DX2DD4 DX2DD5 DX2DD6 DX2DD7 | SEL_L |  |  |
| TXSDQ_X4 | SH | DX2DD4 DX2DD5 DX2DD6 DX2DD7 | SEL_H |  |  |
|  | SL | DX2DD4 DX2DD5 DX2DD6 DX2DD7 | SEL_L |  |  |

Path 2 TNOD/S & TNOD/S_X4 from DMOE_GEN to DX2DD/S & pads

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| DMOE_GEN | TNOD | bdddrdq0 bdddrdq 1 bdddrdq 2 bdddrdq 3 bdddrdq 4 bdddrdq 5 bdddrdq 6 bdddrdq 7 | TNO | T2 |  |
|  | TNOS | bdddrdqs 0 | TNO |  |  |
| DMOE_GEN_X4 | TNOD | bdddrdq 4 bdddrdq 5 bdddrdq 6 bdddrdq 7 | TNO |  |  |
|  | TNOS | bdddrdqs 1 | TNO |  |  |
| DMOE_GEN | TNOD | DX2DD0 DX2DD1 DX2DD2 DX2DD3 DX2DD4 DX2DD5 DX2DD6 DX2DD7 | TNOD | T3 |  |
|  | TNOS | DX2DS DX2DS_X4 | TNOS |  |  |
| DMOE_GEN_X4 | TNOD | DX2DD4 DX2DD5 DX2DD6 DX2DD7 | TNOD |  |  |
|  | TNOS | DX2DS_X4 | TNOS |  |  |

Path 3 TNI & TNI_X4 from DQSIEN_GEN to bdddrdq[s]

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| DQSIEN_GEN | TNID | bdddrdq0 bdddrdq 1 bdddrdq 2 bdddrdq 3 bdddrdq 4 bdddrdq 5 bdddrdq 6 bdddrdq 7 | TNI | T4 |  |
|  | TNIS | bdddrdqs 0 | TNI |  |  |
| DMOE_GEN_X4 | TNID | bdddrdq 4 bdddrdq 5 bdddrdq 6 bdddrdq 7 | TNO |  |  |
|  | TNIS | bdddrdqs 1 | TNO |  |  |

Path 4 DQSIEN_GEN header signal to RXDIO

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| DQSIEN_GEN | DQSIEN_HEAD0 | RXDIO0 RXDIO1 RXDIO2 RXDIO3 RXDIO4 RXDIO5 RXDIO6 RXDIO7 | DQSIEN_HEAD0 | T5 |  |
|  | DQSIEN_HEAD2 | RXDIO0 RXDIO1 RXDIO2 RXDIO3 RXDIO4 RXDIO5 RXDIO6 RXDIO7 | DQSIEN_HEAD2 |  |  |
|  | DDWPG2 | RXDIO0 RXDIO1 RXDIO2 RXDIO3 RXDIO4 RXDIO5 RXDIO6 RXDIO7 | DDWPG2 |  |  |
| DQSIEN_GEN_X4 | DQSIEN_HEAD0 | RXDIO4 RXDIO5 RXDIO6 RXDIO7 | DQSIEN_HEAD0 |  |  |
|  | DQSIEN_HEAD2 | RXDIO4 RXDIO5 RXDIO6 RXDIO7 | DQSIEN_HEAD2 |  |  |
|  | DDWPG2 | RXDIO4 RXDIO5 RXDIO6 RXDIO7 | DDWPG2 |  |  |

Path 4 DCLKO/DCLKOP/DCLKD/DCLKS/DCLKDSI & DCLKD_X4 /DCLKS_X4/DCLKDSI_X4 from VCDL_BYTE_S24 to DIO & pads

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| VCDL_BYTE_S24 | DCLKO | DX2DD0 DX2DD1 DX2DD2 DX2DD3 DX2DD4 DX2DD5 DX2DD6 DX2DD7 TXSDQ TXSDQ_X4 DX2DS DX2DS_X4 DMOE_GEN DMOE_GEN_X4 DQSIEN_GEN DQSIEN_GEN_X4 | CLKO | T6 |  |
|  | DCLKO | PHY TOP | DCLKO |  |  |
|  | DCLKOP | TXSDQ TXSDQ_X4 DX2DS DX2DS_X4 DMOE_GEN DMOE_GEN_X4 | CLKOP |  |  |
|  | DCLKD | DX2DD0 DX2DD1 DX2DD2 DX2DD3 DX2DD4 DX2DD5 DX2DD6 DX2DD7 DMOE_GEN DQSIEN_GEN | TXCLK TXCLKD |  |  |
|  | DCLKD_X4 | DX2DD4 DX2DD5 DX2DD6 DX2DD7 DMOE_GEN_X4 DQSIEN_GEN_X4 | TXCLK TXCLKD |  |  |
|  | DCLKS | DX2DS DMOE_GEN | TXCLK TXCLKS |  |  |
|  | DCLKS_X4 | DX2DS_X4 DMOE_GEN_X4 | TXCLK TXCLKS |  |  |
|  | DCLKDSI | DQSIEN_GEN | CLKDSI |  |  |
|  | DCLKDSI_X4 | DQSIEN_GEN_X4 | CLKDSI |  |  |

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| VCDL_BYTE_S24 | DCLKD | bdddrdq 0 bdddrdq 1 bdddrdq 2 bdddrdq 3 bdddrdq 4 bdddrdq 5 bdddrdq 6 bdddrdq 7 | SEL | T7 |  |
|  | DCLKD_X4 | bdddrdq 4 bdddrdq 5 bdddrdq 6 bdddrdq 7 | SEL |  |  |
|  | DCLKS | bdddrdqs 0 | SEL |  |  |
|  | DCLKS_X4 | bdddrdqs 1 | SEL |  |  |

| Begin |  | End |  | Delay | Note |
| --- | --- | --- | --- | --- | --- |
| IP name | Port | IP name | Port |  |  |
| VCDL_BYTE_S24 | DCLKD | DX2DD0 DX2DD1 DX2DD2 DX2DD3 DX2DD4 DX2DD5 DX2DD6 DX2DD7 | TXCLK | T8 |  |
|  | DCLKD_X4 | DX2DD4 DX2DD5 DX2DD6 DX2DD7 | TXCLK |  |  |

External Constraint Requirement(less)

Internal Constraint Requirement

SEL_H/L setup/hold constraint

tck2q+tmux1+tmux2+T1+tmux3<5T/8-VCDL_Jitter_Skew-Tsetup

TNOD/S & TNI header delay requirement

T3 delay as min as possible

T4<300ps

T5<300ps

DCLKD/S constraint

T6=T7

T7<T8

TXDIO internal

Internal Constraint Conclusion Table

Internal timing requirement in Verilog module by FE as:

Actual timing calculation in schematic by CKT as:

VCDL_BYPASS mode

Calibration(less)

Glitch

For TX clock such as DCLKO/DCLKOP/DCLKD/…, when controller changes corresponding phase setting, there would be no glitch at each clock. It should be noted that, the output clock phase doesn’t follow the setting change immediately. It may take another 0.5~2T to change output clock phase to avoid glitch. Case 1 and case 2 are both possible in below picture.

There is no glitch-avoid function for VCDL_PI_DQS, so controller can change VCDL_PI_DQS phase setting when there is no DQS input. If VCDL_PI_DQS phase setting change in Reading Mode, DQS glitch would appear and DDR couldn’t work.

Frequency change

In normal operation, frequency can’t be changed.

When frequency change, DLL_EN must be trigger from 0 to 1.

DDRPHY FloorPlan & BumpMap

Floorplan

Pad assignment

| DQ4 | DQ5 | DQ0 | DQS9N -DQS9P | DQ1 | VCC4 +CAP | DQ6 | DQS0P -DQS0N | DQ7 | DQ2 | DQ3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

BumpMap

Version:T16 DDR Bump Map Ver.0.5 for CMT003

Test(less)